- Introduction

- Drone Physical Model

- Task Objective

- Key performance

- Methodology

2.1 Environment

Game Bounds

The simulation environment is a two-dimensional continuous space where the drone learns to navigate to hit targets on screen. To ensure the drone stays within the visible bounds of the screen, we determine the game bounds as x∈[-0.75, 0.75] and y∈[-0.50, 0.50], based on the coordinate transformation

x=W- W2minH,W,  y=H-H2min⁡(H,W)

where W=720 is screen width, and H=480 is screen height. Flying out of boundaries result in drone crash and episode termination.

Target Design

We implement three modes for target generation, including fixed mode with 4 predetermined target locations for evaluation, random mode with 5 randomly sampled targets for generalisation testing, and curriculum mode with progressively challenging of random targets for training. In random and curriculum modes, targets are sampled randomly within the game bounds with a minimum inter-target distance of 0.2, a minimum 0.15 distance from origin [0, 0], and a minimum 0.1 distance from boundaries.

2.2 Algorithm

To address drone control problem, we explored both tabular and function approximation reinforcement learning (RL) approaches to understand whether the drone's "optimal flight" requires the robustness of an on-policy tabular controller or the fine precision of a continuous model.

Tabular RL

The main advantage of Tabular RL is its interpretability compared to hidden states in neural network. However, it requires discretising the continuous state space into bins and mapping specific thrusts to a set of heuristic actions. The trade-off is between precise control via fine discretisation and manageable Q-table size.

Due to the hard boundary termination, we employ sarsa, an on-policy temporal difference method, that can account for the agent’s ϵ-greedy exploratory behaviour during training, making it more conservative and risk-aware. Sarsa updates the action-value function according to:

Q(st,at)←Q(st,at)+α[rt+1+γQ(st+1,at+1)-Q(st,at)]

Function Approximation with Neural Networks

Approximation RL addresses the limitations of discretisation though it involves more complexity.

2.3 Agent Architecture

2.3.1 Tabular Sarsa

State Space

The drone has pre-defined six dimensions of continuous states, including relative distance to the target dx and dy, velocity vx and vy, pitch angle θ, and angular velocity ω. We discretised dx and dy into 7 bins, and the rest of states in 3 bins. We also introduced a binary danger flag into the state, activated when the drone approached within 0.1 units of game boundary. Together they form a tuple of 7,938 states.

Action Space

We define six macro actions that map to physical thrust (T1,T2). These include:

- HOVER_STABILISE: Hover at current altitude with pitch stabilisation

- TILT_LEFT/RIGHT: Adjusts pitch for lateral movement.

- BOOST_UP/DOWN: Adjusts vertical thrust with pitch stabilisation.

- ARREST_MOTION: Actively damps linear and angular velocities to bring the drone to a halt.

Reward Design

We implement a balanced reward structure comprise of a massive hit reward when drone hits a target, a scaled progress reward when advancing closer to the target than the previous step, a scaled penalty when approaching boundaries, a painful penalty for crashing out of the bounds, and a constant timestep cost to encourage efficiency.

2.4 Training

Curriculum Learning

We employed a curriculum training. At stage 0, it starts from full exploration with randomised Q-table initialisation.  As the agent advances to stage 1 and then to stage 2, exploration rate decreases and target difficulty increases. Advancement between stages is gated by a rolling average of targets hit per episode.

Episode Termination

An episode terminates when one of three conditions is met: (i) successful completion of all targets in the sequence, (ii) drone crashes as moves out of game bounds, or (iii) maximum timestep running out